# Rover 6 Mechatronics project

K Barnsdale.

January 2020

Table of Contents

1	Introduction	2

2	Build and test PCB	4

3	Start writing software on Rover6 -  PCB only	6

4	Build chassis, add motors, battery etc	9

5	Driving and race skills	10

6	Bumper driving	12

7	Multi tasking (optional if time allows)	14

8	Subroutines	15

9	Binary and memory concepts.	16

10	Music and tunes.	17

11	Using the light sensor	18

12	Final programming	20

13	New Concepts (New) and pre-requisites (PR)	23

# Introduction

## House rules

    - Contribution money required next week

    - Cleanup and chairs up always before anyone leaves

    - Safety glasses to be used when soldering and wash hands after soldering

    - Burning plastic = exclusion from class

## Introduction to Mechatronics

- What is Mechatronics ?

- Mechatronics is combining Electronics, Mechanics and Software

- Career opportunities

## Distribute bags or boxes, and instructions

- Names on storage bags

- Handout instructions so they can show parents

- Distribute template file and final programming file to everyone, and make it ‘read only’

## Rover6 intro

Introduction to Rover6 and its micro controller

## Software:

### New introductions

- Introduce the Picaxe development environment on screen

- Introduce template file (see instructions document)

- Always ‘save as’  new work file to separate folder

- Introduce ‘comments’

- Introduce help documents

- Introduce commands ‘High’ , ‘Low’, and multiple nouns (e.g. High Red, Left)

- Introduce the ‘Do/ Loop’.

- Introduce the ‘Pause’.

- Introduce the on screen simulator

### Tasks

- Using High, low, pause. Turn an output ON for half a second, OFF for 1 second, ON for half a second, then OFF.

Sample code for above - flash LED.bas

high red	;turn on LED

pause 500	;pause for half second

low red		;turn off LED

pause 1000	;pause for a second

high red	;turn on LED

pause 500	;pause for half second

low red		;turn off LED

- Note - we can include two outputs (green and red) in the same command  (low green,red)saving space. Later we will use this to turn motors and LEDs on and off together.

Sample code for above - Cycle LEDS.bas

do

high red		;turn on red  LED

pause 1000		;pause for 1 second

high green		;turn on green LED

pause 500		;pause for half a second

low green, red	;turn off both

pause 200		;pause for 200 milli seconds

loop		;go back and do it again

## Extension work

- Simulate a traffic light, with timing of three colours

- Sequence the LED outputs with one LED on, shifting along the line

- Find ‘pause’  and ‘do’ commands  in help documents

# Build and test PCB

## Hardware:

Start building PCB, using instructions

- Soldering rules

- Discover the parts in the storage cabinet, and components drawers.

- Component identification

- Component polarity

- Only one component at a time

- Soldering instruction and practice

- All components as low down on board as possible

- Explain USB and Light sensor fitting details

## Test PCB hardware

Thorough visual check by teacher (Joints, bridges, polarity etc)

## Instructions on first programming

- Use instructions document for guidance

- Plug in USB cable to check red light on USB module.

- Setup ‘COM port’ number (not AXE027) (see instructions document)

- Click ‘Check Picaxe type connected’ to confirm connection.

- Setup ‘PICAXE type’ to be PICAXE 18M2 (see instructions document)

- Review downloading hang ups and errors. See Rescue Remedies in file ‘Rover 6 Picaxe Rescue remedies’

- Use the simple test program ‘PCB check.bas’  in instructions document to test finished PCB

## Software (if time)

### Revise (pre requisites)

- PICAXE  editor screen layout

- Use of Template

- ‘save as’ new program file to separate folder

- Comments

- High , Low, and multiple nouns (e.g. High Red, Left)

- Do/ Loop

- Pause

### New introductions

- ‘Pulsout’ command

### Tasks

- Flash an LED or three, or copy/paste PCB test code from instructions.

- Try different timing

- Try simulating traffic lights

- Try sequence red yellow green  yellow red etc

Sample code - sequence LEDs.bas

do

high red		;turn ON red  LED

pause 100		;pause for 100 milli second

low red		;turn OFF red LED

high yellow	;turn on yellow LED

pause 100

low yellow

high green		;turn on green LED

pause 100

low green

loop			;go back and do it again

- Using Pulsout creates a pulse on the output pin. The number is the pulse time in 10’s of microseconds. (You can subtract two zeros to get the number of milliseconds)

Sample code –flash LEDs using ‘pulsout’  - pulsout flash.bas

do

pulsout red, 10000 	;flash red for 100 milli sec

pulsout yellow, 10000	;flash yellow for 100 milli sec

pulsout green, 10000	;flash green for 100 milli sec

pulsout yellow, 10000

loop			;go back and do it again

## Extension (if time)

- What is the shortest LED pulse your human eye can see?

- More soldering practice, make a structure, or animal, from resistors

# Start writing software on Rover6 -  PCB only

## Software

### Revise (pre requisites)

- Revise downloading process, and what to look for on downloading screen

- Revise downloading hang ups and errors.

- Revise use of template, and ‘save as’ file to separate folder

- Revise high, low, pause, do/loop

- Revise Comments

### New introductions

- If / else/ endif

- Commands – ‘play’ and ‘sound’

- Command ‘sleep’

### Tasks

- If / else/ endif. We test inputs (bumper switches, light sensor) with the IF statement.  A bumper switch input =1 when pressed, a light sensor input=1 when light. The program lines after ‘then’ will run if the condition is true. The program lines after ‘else’ will run if the condition is not true. Every ‘IF’ statement must end with and ‘endif’ statement.

- Using if/endif– turn an LED on using the left bumper switch

Sample code - bumper LED.bas

do

low red		; LED off.

if lbumper=1 then ; If left bumper pressed...

high red	;Red LED on

endif

loop

- Turn red LED on only while right bumper is pressed

Sample code - bumper on off LED.bas

do

If rbumper=1 then	; test if bumper is pressed (=1)

high red		; red LED on if pressed

else

low red		; red LED off if not pressed

endif

loop

- Try exchanging ‘rbumper’ input for light sensor input. Turn red LED on when light is detected

Sample code - light LED.bas

do

If light=1 then	; test if light is seen (=1)

high red		; red LED on if pressed

else

low red		; red LED off if not pressed

endif

loop

- Turn red LED on/off when each bumper is pressed

Sample code - L and R bumper LED.bas

do

If rbumper=1 then

high red	;turn on red LED when right bumper pressed

endif

If lbumper=1 then

low red	;turn off red LED when left bumper pressed

endif

loop

- The ‘Play’ command

Create a  ‘Play’ command with ‘PLAY Spkr, song’ , where  ‘song’ is a number 0,1,2,3

Sample code -  play command.bas

do

play spkr,1	;can choose song 0,1,2,3

loop

- The ‘Sound ‘ command

SOUND Spkr, (note,duration,note,duration...) where ‘note’ is a number between 90 (low) and 120 (high),   ‘duration’ is time in 10msec steps (100=1sec).

Experiment with different notes and note lengths.

Try sound command, two notes, each one second long

Sample code -   sound.bas

do

sound spkr,(90,100,120,100) ;the ‘100’ means notes of one second each

loop

A reaction GAME with If/Else and Sound. Read the comments to find out how it works.

Sample code - reaction.bas

do

high yellow	; turn on LED….

pause 3000	; …. and wait for 3sec

low yellow	; as soon as LED turns off – push the bumper switch!

pause 300	 ; give them time to push the button

if rbumper = 0 then	;if not pushed by now, you are too slow

sound spkr, (120,50,90,50) 	; loser!

else

sound spkr, (90,50,120,50) 	; winner!

endif

loop

#### Sleep

We can put the Rover microcomputer into a deep sleep mode which takes almost no power from the battery. The number in the sleep command is the sleep time in steps of 2.3 seconds

Sample code – using sleep command – sleep.bas

do

pulsout yellow, 1000  ;flash an LED at the start

sleep 10 ; 10 x 2.3 seconds = 23 seconds

play spkr,2

loop

### Extension work

- Add LEDs to the winner / looser sections of reaction game.

- Make two different sounds on left/right bumper button presses

- Use light sensor to change the sound

- Flash LED in morse code

# Build chassis, add motors, battery etc

## Hardware

- Build chassis and motors, mount PCB and wiring. See instructions sheet

## Software (if time)

### Revise (pre requisites)

- Use of Template, ‘save as’ file to separate folder

- Do/loop

- High, low, Pause

### New introductions

- High Left, High right (forward commands for motors)

- High lback, High rback (backward commands for motors)

### Tasks

- Test motors and battery (forward, back, left right). Use test program (motor check.bas) in instructions document, or right your own using highs, lows and pauses, as below.

Sample code to test all motors - motor test.bas

do

high right,left ;both motors on forward

pause 2000	;drive for 2 seconds

low left	;turn off forward left motor, turn to left

pause 2000

high lback	;reverse left motor, spin

pause 2000

low right	;turn off forward right motor, backwards turn

pause 2000

high rback	;reverse right motor, drive backwards

pause 2000

low rback, lback ;turn off all reverse

loop

- Practice turns, spin. Calibrate timing for turns.

- Games (if time)

- Race to the line.

- Race around a square

## Extension work

Try stopping with

a/ Both forward and back outputs ‘Low’

b/ Both forward and back outputs  ‘High’

and see what difference there is in the way it stops.

(Outputs ‘low’ applies a brake to the wheel, outputs ‘high’ allows free wheel)

Sample code – different stopping techniques – way to stop.bas

high right,left ;both motors on forward

pause 1000	;drive for 1 second

low right, left ; brake!

pause 1000

high right,left ;both motors on forward

pause 1000	;drive for 1 second

high lback, rback ; stop!

# Driving and race skills

## Software

### Revise (pre requisites)

- Use of template, ‘save as’ file to separate folder

- Do/loop

- High, low, Pause

### New introductions

NA

### Tasks

- Calibrate speed/distance

- Race to over the line and return by reverse

- Calibrate turns and /or spin (how long should one wheel be stopped?)

- Race to line and turn around to return

- Rally to line (accurate timing)

- Use PWM to control speed

Sample code - to drive ahead and reverse – note it runs once - NO ‘do / loop’ here!  race.bas

high left, right, red, green ; all ON

pause 5000			;drive for a bit. You work out how long

low left, right, red, green ; all forwards OFF

high lback,rback, yellow 	; all reverse

pause 5000			;drive for a bit. You work out how long

low lback, rback, yellow 	; all OFF

Sample code to do a turn and a spin – turn and spin.bas

do

; turn by stopping one wheel

high left, right, red, green

pause 2000		; drive for a bit

low left, red 	; turn off left motor

pause 1550

high left, red

pause 2000		; forward again for 2sec

; turn by spin

low left, red	; turn off forward drive

high lback, yellow 	; turn on backward left

pause 1000			; spin left for 1 sec

low lback, yellow 		; turn OFF backward left

high left, red		; turn on forward left

loop

## Extension work

Drive a square figure of eight

## Extra features

- Use buttons to control PWM up and down

Sample - This adjusts the speed by pressing the bumpers – PWM test.bas

pwmout left, 50, 100	;setup PWM duty ratio

pwmout right, 50, 100

do

pwmduty left,b0	;change PWM ratio

pwmduty right,b0

low red

if lbumper=1 then ;test for button press

b0=b0+1

high red

;pause 100 debug	;optional readout of PWM duty b0

endif

loop

# Bumper driving

## Software

### Revise (pre requisites)

- Revise comments, template discipline

- Revise Do/loop, High, Low, Pause

- Revise If/Else/Endif

### New introductions

- Introduce FOR/NEXT loop

- IF/endif to detect bumper activation and reverse/turn away

- For /Next loop to bumper driving to add ‘rescue code’ when Rover gets stuck

### Tasks

Bumper driving

Using the bumper switches, use IF/ENDIF to detect when one has been hit, reverse and turn away. Note the use of LEDs with the motors to show what is going on. The working of this program can then be ‘seen’ with just the USB power connected. Note the reversals do not shut off the reversing motor, that is done when it gets back to the start of the loop.

Sample code for bumper driving, bumper drive.bas

do

low rback,lback,red,green ;turn all off

high right, left , yellow ;drive forward

if lbumper=1 then	; test bumper

low left, right ;stop!

high rback, green ; reverse opposite wheel

pause 500	;do this for a while

endif

if rbumper=1 then	; test bumper

low left, right ;stop!

high lback, red ; reverse opposite wheel

pause 500	;do this for a while

endif

loop

FOR/NEXT loop

FOR/NEXT loop used in a simple case. This goes around a loop five times and flashes an LED each time.  Try simulating it to see counter (b0) change. Don’t put this inside a do/loop, otherwise you will never see it finish!

Sample code for For/Next loop. This flashes red LED five times – for next.bas

for b0=1 to 5 ;do the next bit five times

high red	;flash an LED

pause 100

low red

pause 100

next b0		;do it again if not finished

## Extra features

Incorporating the FOR/NEXT function into the bumper driving program.

Sometimes the Rover will get stuck against a wall. A simple ‘reverse and turn’ after a set time will rescue it. The main loop is run 2000 times (for/next loop) before it executes a reversal.

Sample code for bumper driving, bumper drive 2.bas

do

for w0=1 to 2000	;do this loop 2000 times

low rback,lback,red,green ;turn all off

high right, left , yellow ;drive forward

if lbumper=1 then	; test bumper

low left, right ;stop!

high rback, green ; reverse opposite wheel

pause 500	;do this for a while

endif

if rbumper=1 then	; test bumper

low left, right ;stop!

high lback, red ; reverse opposite wheel

pause 500	;do this for a while

endif

next w0

; the next bit rescues the Rover if it gets stuck

low left, right, yellow; intermittent direction change

high lback;

pause 500	; short reversal

loop

## Extension

- In the FOR/NEXT example, flash a green light when the it has finished, then put the whole thing in a DO/LOOP

- In a FOR/NEXT loop, try counting down.

- In the Bumper driving, make the rescue code only run when it hasn’t seen a bumper activation for a while.

# Multi tasking (optional if time allows)

## Software

### Revise (pre requisites)

- Revise comments, template discipline

- Revise Do/loop, High, Low, Pause

- Revise If/Else/Endif

- Pulsout,  which uses a time in 10 microsecond units (eg 5000 = 50mSec)

### New introductions

Introduce multitasking – (start1: start2: etc)

### Tasks

Flash three LEDs with multi tasking.

Watching the flashes, ask your friend to guess the fastest/slowest?

Sample code to flash three LEDs with multi tasking- multitask.bas

start1: ; First task

do

pulsout red, 5000 ; pulse LED for 50mSec

pause 1000   ; Each task has a slightly different delay

loop

start2: ; second task

do

pulsout green, 5000 ; pulse LED for 50mSec

pause 1100

loop

start3:  ; third task

do

pulsout yellow, 5000  ; pulse LED for 50mSec

pause 1200

loop

Sample code added to bumper program, to flash Yellow when driving – bumper flash yellow.bas

start2:

do

pulsout yellow, 4000

pause 500

loop

## Extension

TBA

# Subroutines

## Software

### Revise (pre requisites)

- Revise comments, template discipline

- Revise Do/loop, High, Low, Pause

- Revise turns / spin calibration

### New introductions

Introduce subroutines - ‘Gosub’, a separate code routine, terminated by ‘Return’.

### Tasks

- Subroutines are used to perform repetitive tasks. If we create subroutines to turn 90deg left and right, the time calibration can be adjusted in just one place. These can then be used by a pattern driving program.

- Drive a square

- Drive a figure of eight

Sample code -  subroutines to turn left and right – sub pattern.bas

;subroutine for left turn

turnl:		;name of subroutine

low left, red

pause 420		;time for a left turn

high left,red

return		;go back to main program

;subroutine for right turn

turnr:		;name of subroutine

low right, green

pause 420		;time for a right turn

high right, green

return		;go back to main program

Sample code – drive a figure of eight using the subroutines above – sub pattern.bas

do        ;program for figure of eight

high left,right,red,green

gosub turnl:		;turn left

pause 1000		;drive for 1 sec

gosub turnl:		;turn left

pause 1000		;drive for 1 sec

gosub turnl:		;turn left

pause 1000		;drive for 1 sec

gosub turnl:		;turn left

pause 1000		;drive for 1 sec

gosub turnr:		;turn right

pause 1000		;drive for 1 sec

gosub turnr:		;turn right

pause 1000		;drive for 1 sec

gosub turnr:		;turn right

pause 1000		;drive for 1 sec

gosub turnr:		;turn right

pause 1000		;drive for 1 sec

loop

# Binary and memory concepts.

## Software

### Revise (pre requisites)

- Revise comments, template discipline , ‘save as’ file to seperate folder.

- Revise Do/loop, High, Low, Pause

### New introductions

- Introduce bytes and word memory,  memory locations b0, b1, etc

- Introduce ‘inc’  increment (add one)

- Introduce binary counting and 0-255 numbers

- Binary count to the port connected to the LEDs  (pinsb)

### Tasks

- Use ‘binary chair game’ to demonstrate binary values, and multiply/divide by 2 by shifting.

- Binary count on LEDs

Sample code -  Binary count on LEDs. Count up to seven? – binary.bas

low red,green,yellow ; make LED pins outputs

do

inc pinsb 	;add one to the LEDs

pause 1000

loop

- Count up and down  in binary using bumper switches, watch it on the LEDs

Sample code -  Binary count using bumpers – bumper binary.bas

Low red,green,yellow ; make LED pins outputs

do

if lbumper=1 then

b0=b0+1	;add one if left bumper

endif

if rbumper=1 then

b0=b0-1	;subtract one if right bumper

endif

pinsb=b0 ;send the result to the LEDs

pause 200

loop

# Music and tunes.

Not much in this section, but playing ringtones can use a lot of time. Maybe combine with another topic.

## Software

### Revise (pre requisites)

- Revise comments, template discipline

- Revise Do/loop

- Sound command with memory control

### New introductions

- Tune wizard

- Tune editor

### Tasks

Tune wizard

- Open tune wizard , under the PICAXE menu tab

- Select output B.7 (spkr) as the ‘Piezo pin’

- Select ‘ LED flash’ output (Yellow is B.1)

- Use ‘Find a tune’ link to access tunes website

- Download ‘mixed tunes 1’ and unzip to new tunes folder on desktop. (Avoid ‘TV tunes’ and ‘Xmas tunes ‘avoid for now, they are a different format)

- Use ‘Open RTTTL.txt file’ to select tune, ‘generate the BASIC code – yes’ and no need to generate the .wav file.

- Use ‘Copy button ‘ to copy/paste  to new program template

Sample code –tune ‘abdelaize.txt’ should look like this – abdelaize.bas

do

tune B.7, 4,%00000010,($F2,$F5,$F9,$02,$44,$45,$47,$45,

$44,$42,$C1,$09,$42,$45,$49,$45,$02,$C9,$07,$40,$44,$47,

$44,$00,$C9,$05,$7B,$42,$45,$42,$3B,$C7,$04,$79,$41,$44,

$40,$39,$C5,$44,$45,$44,$42,$01,$05,$44,$45,$44,$42,$39,

$02,$41,$42,$44,$42,$C2)

loop

Tune editor

Using the editor in the tune wizard, try to write a tune of your own in the tune editor!

# Using the light sensor

## Software

### Revise (pre requisites)

- Revise comments, template discipline

- Revise Do/loop

- Sound command with memory control

- Memory and bytes

- If then/ else/ endif

### New introductions

- IF/Else/endif using the light sensor as an input

- ‘ReadADC’ using the ‘Vlight’ input

### Tasks

- Light Tunes. Read the light level using ‘Readadc’ command, and make a sound dependant on light level . Sounds need a number between 0 to 127 whereas light levels can be 0 to 255, so we divide light level by 2. (b0 and b1 are temporary memory stores).

Sample code – Light Tunes.bas

do

readadc Vlight, b0  ; get light level into byte 0 (b0)

b1=b0/2		; make it <127

sound spkr,(b1,10)	;make a sound with the result

loop

- Read light value using ‘readadc’ command and show value on LEDs. As the light level can go up to 255, we need to divide the number by 32 to get it back to the 0-7 range for the three LEDs. (b0 and b1 are temporary memory stores).

Sample code -  Show light sensor readings on the LEDs – light on LEDs.bas

Low red,green,yellow	;make all LEDs ports as outputs

do

readadc Vlight, b0	;fetch light reading

b1=b0/32			;divide by 32 so the LEDs can show

pinsb=b1			;send it to the LEDs

loop

- Follow light. This will require light beam in a dark area. Use the LEDs to show where light and dark areas are.

Sample code – follow edge of light beam – light follow.bas

do

if light=1 then	;test if light

high left,red ;turn right

low right,green

else

high right,green ;turn left

low left,red

endif

loop

## Extension work

- Dairy door chime - Detect changes in light and make a sound when it changes

Sample code - door chime.bas

do

low red,green	; turn off activity leds

B2=B1+1	;make current light+1  the upper limit

B3=B1-1	;make current light-1  the lower limit

readadc vlight,B1	; read current light level

if B1>B2 then 	;test if light greater than upper limit

high red

sound spkr,(90,10)

endif

if B1<B3 then 	;test if light less than lower limit

high green

sound spkr,(100,10)

endif

## loop

# Final programming

## Games day. White board patterns.

### Software

### Revise (pre requisites)

- Revise comments, template discipline

- Revise Do/loop, High, Low, Pause

### New introductions

NA

### Tasks

Attach whiteboard pen to back end of Rover with bulldog clip or bluetac.

Drive distorted circles on the whiteboard . (Turn more than go straight, to keep it small!)

Sample code - Driving to make orange segments pattern – drive pattern.bas

do	;orange segments

high left,red		;turn right….

pause 2500			;…for 2.5sec

low left,red		;then stop.

high right,green		;turn left …

pause 1500			;…for 1.5sec

low right,green		;then stop

loop

## Options for ‘take home’ programs

- Bumper programme

- Light tunes

- Light follower

- Door chimes

Three function program to take home  Three prog.bas

; Rover 6

;    Filename:   Three prog

;    Function:  Bumper, light follow, light tune

; *******************************

#no_data

#PICAXE 18m2

;setfreq m32

symbol left = b.3 ; Right forward

symbol right = b.6 ; Left forward

symbol lback = b.4 ; Right reverse

symbol rback = b.5 ; Left reverse

symbol spkr = b.7 ; Speaker

symbol green = b.0 ; LED Green

symbol yellow = b.1 ; LED Yellow

symbol red = b.2 	; LED Red

symbol light = pinc.0 ; Light/dark sensor

symbol Vlight = c.0 ; Variable light sensor

symbol Lbumper = pinc.1 ; Bumper switch left

symbol Rbumper = pinc.2 ; Bumper switch right

symbol nUSB = pinc.5 ; USB not active

high yellow		;power light

; press a bumper whilst you switch on

; if left bumper pressed then run light tune program

if lbumper = 1 then lighttune

;if right bumper pressed then run light follow program

if rbumper = 1 then lightfollow

;if no bumper pressed then run bumper driving program

bumper:

pause 1000

do

for w0=1 to 2000

low rback,lback,red, green

high right, left, yellow ;drive fwd

if lbumper=1 then		;test bumper

low left, right 	;stop!

high rback, green	; reverse opposite wheel

pause 500 		;do this for a while

endif

if rbumper=1 then

low left, right	;stop!

high lback, red	; reverse opposite wheel

pause 500		;do this for a while

endif

next w0

low left, right ; intermittent direction change

high lback

low yellow ;

pause 500

loop

; Program to follow light beam

lightfollow:

pause 1000

do

if light=1 then

high right,green 	; right on

low left, red 	; left off

else

low right,green 	;right off

high left, red 	; left on

endif

loop

; Program to play sounds from light

lighttune:

low green, yellow, red

do

readadc vlight,b0	;get the light level

pinsb=b0/32	;send a version to the LEDs

b0=b0/2	;divide light by 2 to play the sound

sound spkr, (b0,10)

loop

# New Concepts (New) and pre-requisites (PR)

| Concept | Intro | Build PCB | PCB S/W | Build chassis | Driving | Bumper | Multitask | Subs | Binary | Music | light |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prog editor, Comments, Template, Low,High, MultiNoun, Pause, Do/loop | NEW | PR | PR | PR | PR | PR | PR | PR | PR | PR | PR |
| Prog Download |  | NEW | PR | PR | PR | PR | PR | PR | PR | PR | PR |
| Pulsout |  | NEW |  |  |  |  | PR |  |  |  |  |
| If/else/endif |  |  | NEW | PR | PR | PR | PR | PR |  |  | PR |
| Play,Sound |  |  | NEW |  |  |  |  |  |  | PR | PR |
| Sleep |  |  | NEW |  |  |  |  |  |  |  |  |
| Reverse |  |  |  | NEW |  |  |  |  |  |  |  |
| For/Next |  |  |  |  |  | NEW |  |  |  |  |  |
| Multitask |  |  |  |  |  |  | NEW |  |  |  |  |
| Subroutine |  |  |  |  |  |  |  | NEW |  |  |  |
| BinaRy, Bytes, Memory |  |  |  |  |  |  |  |  | NEW |  | PR |
| Increment, Decrement |  |  |  |  |  |  |  |  | NEW |  |  |
| Tune Wiz |  |  |  |  |  |  |  |  |  | NEW |  |
| ReadADC |  |  |  |  |  |  |  |  |  |  | NEW |